# Traffic Data

User Guide to create Traffic Data, v.01

© COPYRIGHT CONSAT 2020

All rights reserved.

The content of this document may be subject to revision without notice. Consat has no liability for typing errors in this document.

No part of this document may be copied, distributed, transmitted, transcribed, stored in a retrieval system, or translated into any human or computer language without the prior written permission of Consat.

Table of Contents

1. Introduction	5

1.1 Transmodel Lexicon	5

2. Our Pretend System	5

3. Required Files	5

4. My Stop Points.xlsx	6

5. List of Stop Point.xlsx	6

5.1 Table Content	7

5.1.1. PIJATTRIBUTES	8

5.2 Line Silverstone – Tab 8	9

5.3 Line 202 – Tab 202	10

6. Line and Route Files	11

6.1 File 8_20160112_20160428_1	13

6.2 File 202_20160112_20160428_2	13

Document Version

| Version | Author | Issue Date | Comment |
| --- | --- | --- | --- |
| 1.0 | J. B. | September 9, 2016 | First release |
| 2.0 | J.L. | October 30, 2020 | Adding PIJATTRIBUTES |

# Introduction

Traffic data is very specific and the way to generate (or create) the Excel files is important in order for everything to run smoothly.

Terms can differ between Europe and North America. For the purpose of this document, European terms will be used.

Included with this guide are 4 Excel files:

List of Stop Points.xlsx: required for the actual traffic data import

MyStopPoints: only for internal use, is not required for the actual traffic data import

2 route files.xlsx (202_... and 8_...): required for the actual traffic data import

## Transmodel Lexicon

| Term | Definition |
| --- | --- |
| Route | An ordered list of stop points, links and destination. E.g. Route 1 and Route 2. |
| Line | A group of routes with a designation. A line usually has two routes, one for each direction. E.g. Line 3 is made of both Route 1 and Route 2. However, a line can have more routes, when there are small variations in a direction. |
| Journey | A route with a date, a start time and an end time. Each journey is unique and has its own ID. |

# Our Pretend System

Our pretend system has 2 lines.

Line Silverstone, which has two routes

Route 1 towards Club

  - Route 2 towards Abbey

Line 202, which has four routes

  - Route 1 towards North01

  - Route 3 towards North02

  - Route 2 towards South01

  - Route 4 towards South02

# Required Files

Before you start, make sure you have a file with a list of stop points, stop areas, and their UNIQUE ID. This file is more for your own internal use and is not needed for importing traffic data.

The following Excel files are required for the import:

List of Stop Points.xlsx.

One file for each route, for each line.

  - In our pretend system, that means we will have 6 files for the routes.

This mean that, in total, our pretend system will provide 7 Excel files (note that the attached examples only provide 2 route files, one for each line).

# My Stop Points.xlsx

Most systems use stop areas and stop points, where an area can comprise one or more points. Usually, an area will contain the stop points for both directions.

E.g. Centrum is the stop area. Centrum A and Centrum B are two stop points, where vehicles stop for passengers to alight or disembark.

Each stop point area has its own UNIQUE ID. Each stop point has also its own UNIQUE ID.

E.g. Centrum has the ID 255. Centrum A has 25501, while Centrum B is 25502.

# List of Stop Point.xlsx

This file contains all the stop points, listed by LINES and ROUTES.

REQUIRED

One tab per line.

  - In our example, that means we have 2 tabs; one for Line Silverstone and one for Line 202.

Each tab must be named with NUMBERS only, never letters.

  - In our example, that means we must assign numbers to Silverstone, which we decided would be 8. Note that you can have the line’s name in the file, to more easily link the name to the number (field LINENAME).

  - Our tabs will be: 8 and 202.

Each tab lists ALL routes, one after the other, with ALL stop points.

  - In our example, tab 8 will have 2 routes, and tab 202 will have 4 routes.

Each route has a direction number. These numbers are unique to a line, but each line can have a Direction 1, Direction 2, etc. They usually do.

  - TIPS: A line usually has 2 destinations (back and forth). We suggest that you name all routes going forth with an uneven number (1, 3, 5, …) and all routes going back with an even number (2, 4, 6, …)

    - Line 202, North01 will have Direction 1

    - Line 202, North02 will have Direction 3

    - Line 202, South01 will have Direction 2

    - Line 202, South02 will have Direction 4

    - Line Silverstone, Club will have Direction 1

    - Line Silverstone, Abbey will have Direction 2

Distance is NOT required, but is always useful to have if the information is available.

EXTERNAL ID is UNIQUE through the whole system. You cannot have two stop point with the same EXTERNAL ID.

## Table Content

| Header | Description | Examples |
| --- | --- | --- |
| * LINENAME | The name of the line, useful if you have lines with Colours to associate its number to its name. | Blue, Silverstone |
| Direction | A number associated to the direction. Must be unique WITHIN a tab. Can be repeated in OTHER tabs. | 1, 2, 3, 4, … |
| Destination | The destination of the route, often its last stop point. | Club, Abbey |
| * Via Destination | A via destination that will appear on the vehicle’s displays until the stop point is passed. | Luffield, Senna |
| External ID | A UNIQUE number associated to the stop point. | 25501, 25502 |
| * Stop point position | The position of a stop point in a stop area. | A, B. 1, 2 |
| Stop point (stop name) | The name of the stop point. Should be the same as the stop area name. | Brooklands, Maple Square. |
| External stop area ID | A UNIQUE number associated to the stop area. | 255, 261 |
| Stop area (name) | The name of the stop area. Should be the same as the stop point name. | Brooklands, Maple Square. |
| Timing point | Yes/no to identify timing points. | Yes, No |
| Latitude | The Latitude coordinates of the stop point. | 59.31027 |
| Longitude | The Longitude coordinates of the stop point. | 18.0253 |
| * Distance | The cumulative distance from one point to another, in meters. | 689, 5890 |
| PIJATTRIBUTES | Attribute linked to a stop point AND a route. | Parcel pick-up, alighting request, boarding request |

* The fields marked by an * are not mandatory. Everything is REQUIRED to be filled with proper information.

### PIJATTRIBUTES

The Parcel Attributes has 4 options. Note that the spelling has to be in the format WordWordWord in the Excel file.

Empty: Nothing to denote

ParcelPickupPoint

  - The stop point is also a freight terminal where the vehicle needs to pick up a parcel.

BoardingRequiresDriverArrangement

  - The driver must be informed that passengers have made a boarding request at this stop point, otherwise the default behaviour is to drive past it.

  - Passengers must request in advance that they want to board at this stop point. E.g. they can call a certain number or book online.

AlightingRequiresDriverArrangement

  - Same as above, but regarding alighting.

## Line Silverstone – Tab 8

Silverstone has 2 routes, each of them shown with its own section.

| LINENAME | Silverstone |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DIRECTION | 1 |  |  |  |  |  |  |  |  |  |
| DESTINATION | VIA DESTINATION | EXTERNAL ID | STOP POINT POSITION | STOP POINT (STOP NAME) | EXTERNAL STOP AREA ID | STOP AREA (NAME) | TIMING POINT | LATITUDE | LONGITUDE | DISTANCE |
| Club |  | 25001 | A | Abbey | 250 | Abbey | yes | 59.31027 | 18.0253 | 0 |
| Club |  | 26401 | A | The Loop | 264 | The Loop | no | 59.31599 | 18.034 | 689 |
| Club |  | 25101 | A | Aintree | 251 | Aintree | yes | 59.32839 | 18.02214 | 812 |
| Club |  | 25401 | A | Brooklands | 254 | Brooklands | no | 59.33233 | 18.028768 | 1268 |
| Club |  | 25801 | A | Luffield | 258 | Luffield | yes | 59.34563 | 18.03412 | 2057 |
| Club |  | 26501 | A | Woodcote | 265 | Woodcote | no | 59.34785 | 18.03035 | 3068 |
| Club |  | 25301 | A | Becketts | 253 | Becketts | yes | 59.35315 | 18.03177 | 4281 |
| Club |  | 25601 | A | Chapel | 256 | Chapel | no | 59.65126 | 17.931093 | 5169 |
| Club |  | 26301 | A | Stowe | 263 | Stowe | no | 59.647777 | 17.928609 | 5569 |
| Club |  | 26001 | A | Paddocks | 260 | Paddocks | yes | 59.644811 | 17.928427 | 5890 |
| DIRECTION | 2 |  |  |  |  |  |  |  |  |  |
| DESTINATION | VIA DESTINATION | EXTERNAL ID | STOP POINT POSITION | STOP POINT (STOP NAME) | EXTERNAL STOP AREA ID | STOP AREA (NAME) | TIMING POINT | LATITUDE | LONGITUDE | DISTANCE |
| Abbey | Luffield | 26002 | B | Paddocks | 260 | Paddocks | yes | 59.651723 | 17.933131 | 0 |
| Abbey | Luffield | 26302 | B | Stowe | 263 | Stowe | no | 59.646974 | 17.928636 | 321 |
| Abbey | Luffield | 25602 | B | Chapel | 256 | Chapel | no | 59.644442 | 17.929087 | 721 |
| Abbey | Luffield | 25302 | B | Becketts | 253 | Becketts | yes | 59.35332 | 18.03164 | 1609 |
| Abbey | Luffield | 26502 | B | Woodcote | 265 | Woodcote | no | 59.34816 | 18.02911 | 2822 |
| Abbey | Luffield | 25802 | B | Luffield | 258 | Luffield | yes | 59.34549 | 18.03392 | 3833 |
| Abbey |  | 25402 | B | Brooklands | 254 | Brooklands | no | 59.332412 | 18.02864 | 4622 |
| Abbey |  | 25102 | B | Aintree | 251 | Aintree | yes | 59.32786 | 18.02194 | 5078 |
| Abbey |  | 26402 | B | The Loop | 264 | The Loop | no | 59.31495 | 18.03401 | 5201 |
| Abbey |  | 25002 | B | Abbey | 250 | Abbey | yes | 59.31027 | 18.0253 | 5890 |

## Line 202 – Tab 202

Line 202 has four routes, so we need to show each of them, with their own stop points and data. Notice how each route has its own number in the DIRECTION field.

| DIRECTION | 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESTINATION | VIA DESTINATION | EXTERNAL ID | STOP POINT POSITION | STOP POINT (STOP NAME) | EXTERNAL STOP AREA ID | STOP AREA (NAME) | TIMING POINT | LATITUDE | LONGITUDE | DISTANCE |
| Maple Square |  | 25201 | A | Arena | 252 | Arena | yes | 59.31027 | 18.0253 | 0 |
| Maple Square |  | 26201 | A | St. Joseph | 262 | St. Joseph | no | 59.31599 | 18.034 | 689 |
| Maple Square |  | 25101 | A | Aintree | 251 | Aintree | yes | 59.32839 | 18.02214 | 812 |
| Maple Square |  | 25501 | A | Centrum | 255 | Centrum | no | 59.33233 | 18.028768 | 1268 |
| Maple Square |  | 25901 | A | Maple Square | 259 | Maple Square | yes | 59.34563 | 18.03412 | 2057 |
| DIRECTION | 2 |  |  |  |  |  |  |  |  |  |
| DESTINATION | VIA DESTINATION | EXTERNAL ID | STOP POINT POSITION | STOP POINT (STOP NAME) | EXTERNAL STOP AREA ID | STOP AREA (NAME) | TIMING POINT | LATITUDE | LONGITUDE | DISTANCE |
| Arena |  | 25902 | B | Maple Square | 259 | Maple Square | yes | 59.34563 | 18.03412 | 0 |
| Arena |  | 25502 | B | Centrum | 255 | Centrum | no | 59.33233 | 18.028768 | 321 |
| Arena |  | 25102 | B | Aintree | 251 | Aintree | no | 59.32839 | 18.02214 | 721 |
| Arena |  | 26202 | B | St. Joseph | 262 | St. Joseph | yes | 59.31599 | 18.034 | 1609 |
| Arena |  | 25202 | B | Arena | 252 | Arena | no | 59.31027 | 18.0253 | 2822 |
| DIRECTION | 3 |  |  |  |  |  |  |  |  |  |
| DESTINATION | VIA DESTINATION | EXTERNAL ID | STOP POINT POSITION | STOP POINT (STOP NAME) | EXTERNAL STOP AREA ID | STOP AREA (NAME) | TIMING POINT | LATITUDE | LONGITUDE | DISTANCE |
| Maple Square | Senna | 25201 | A | Arena | 252 | Arena | yes | 59.31027 | 18.0253 | 0 |
| Maple Square | Senna | 26201 | A | St. Joseph | 262 | St. Joseph | no | 59.31599 | 18.034 | 689 |
| Maple Square | Senna | 26101 | A | Senna | 261 | Senna | yes | 59.332412 | 18.02864 | 943 |
| Maple Square |  | 25101 | A | Aintree | 251 | Aintree | yes | 59.32839 | 18.02214 | 1066 |
| Maple Square |  | 25501 | A | Centrum | 255 | Centrum | no | 59.33233 | 18.028768 | 1522 |
| Maple Square |  | 25901 | A | Maple Square | 259 | Maple Square | yes | 59.34563 | 18.03412 | 2311 |
| DIRECTION | 4 |  |  |  |  |  |  |  |  |  |
| DESTINATION | VIA DESTINATION | EXTERNAL ID | STOP POINT POSITION | STOP POINT (STOP NAME) | EXTERNAL STOP AREA ID | STOP AREA (NAME) | TIMING POINT | LATITUDE | LONGITUDE | DISTANCE |
| Arena | Senna | 25902 | B | Maple Square | 259 | Maple Square | yes | 59.34563 | 18.03412 | 0 |
| Arena | Senna | 25502 | B | Centrum | 255 | Centrum | no | 59.33233 | 18.028768 | 321 |
| Arena | Senna | 25102 | B | Aintree | 251 | Aintree | no | 59.32839 | 18.02214 | 721 |
| Arena | Senna | 26102 | B | Senna | 261 | Senna | yes | 59.332412 | 18.02864 | 975 |
| Arena |  | 26202 | B | St. Joseph | 262 | St. Joseph | yes | 59.31599 | 18.034 | 1863 |
| Arena |  | 25202 | B | Arena | 252 | Arena | no | 59.31027 | 18.0253 | 3076 |

# Line and Route Files

These files contain the timetable for all the journeys (trips) servicing this specific route. It lists ONLY the timing stop points.

Each route requires its own file. The name of the file can be anything, though we recommend using the same logic, to make the files easy to identify. A suggestion is: LineNumber_FromDate_ToDate_RouteDirectionNumber. In our example, we would have the following 6 Excel files:

- 8_20160112_20160428_1 (example provided)

- 8_20160112_20160428_2

- 202_20160112_20160428_1

- 202_20160112_20160428_2 (example provided)

- 202_20160112_20160428_3

- 202_20160112_20160428_1

Each file should contain 7 tabs; one for each day of the week.

- Note that you can group some tabs together. The following list contains all allowed names for the tabs. Note that the spelling is important.

  - Monday

  - Tuesday

  - Wednesday

  - Thursday

  - Friday

  - Saturday

  - Sunday

  - Tuesday-Wednesday

  - Monday-Thursday

  - Weekdays

  - Everyday

  - Vacation

Do not change any headers or categories. The lists were created a while ago using a Brazilian system, hence the few Portuguese words.

The fields you should update are highlight in blue and shown in the image below.

4

3

2

1

- The first number should be your line number. In this example, Silverstone has the line number 8. This refers to the right tab in the List of Stop Points.xlsx. The second number is the route for this file. In this case, we are looking at the route with Direction 1.

- Enter your company’s name. Also enter the validity dates, both from and to. This is important for the system to know when the data is valid. It can also be used for creating more complex data sheet, where specific dates can have their own schedule (e.g. Christmas, Labour Day, etc.). More than one validity period can be entered. In that case, add more rows and make sure that they don’t overlap in time.

- One column per Timing Point. Make sure to only enter the timing point and not all the stop points.

- List every journey, along with which block it belongs to. Then for each journey, list the timetable time for each Timing Point.

## File 8_20160112_20160428_1

This shows my route 1 for line Silverstone (# 8).

## File 202_20160112_20160428_2

This shows my route 2 for line 202.